|  | Engine 617cc Tohatsu 2 strokes, water-cooled gasoline engine Authorized output 40.8PS (30kW) Pump performance 1700 liter / min. at 0.4MPa 1300 liter / min. at 0.8MPa 950 liter / min. at 1.0MPa Dry weight 85kg |
| --- | --- |

## Features of VC72AS

1. Light weight and compact 
Engine and pump castings are made of anti-corrosive aluminum-alloy to reduce weight and prolong 
working life. Compact unit in its class fits existing locker space, also can be carried by 2 firemen.

2. High pump performance 
Both engine and pump are designed and developed specifically for fire pumps by Tohatsu, ensuring 
higher pump performance for its class.

3. Reliable engine with oil injection/auto mixing oil delivery system 
The engine of VC72AS is originally designed by Tohatsu for VC52/72AS models. Equipped with oil 
pump, gasoline and engine oil are mixed automatically at a precise ratio to engine speed without mixing 
fuel and oil by operator.

4. Priming 
Quick priming is provided by operating Rotary Vane Vacuum Pump. Prime 3 meters in 6 seconds.

5. Centralized control 
All gauges, throttle dial, control switches, and other warning lamps are grouped on the control panel for 
easy operation.

6. Cooling water recirculation system 
Engine cooling water is returned to the pump eliminating discharge outside of the pump, and 
contamination of surrounding area.

7. Unique auto choke carburetor 
This device helps engine start easier in all seasonal temperature conditions.

8. Engine overheat protection sensor 
Overheat protection is provided on this model to safeguard the engine from cooling water loss, 
and to protect engine when water is not discharging. This device stops the engine automatically 
when engine temperature reaches 80℃. Reset switch is provided to release the sensor after 
it stops the engine.

9. Stainless steel fuel tank 
Integrated fuel tank with large capacity of 18liters ensures the fuel supply will last for 1.5 hours at least 
at maximum throttle opening and load.

10. 180degree rotating discharge port

11. Electric starter with back up recoil starter 
An electric starter and back-up recoil starter are installed to guarantee starting engine under any 
conditions.

Accessories

1. Battery charger

2. Floodlight projector

3. Plastic toolbox

Adapters

Some different type of adapters for suction port and discharge ports are available.

| Tohatsu portable fire pump Model VC72AS Specifications |  |  |  |
| --- | --- | --- | --- |
| Model: |  | VC72AS |  |
| Engine: | Type: | Tohatsu 2-stroke, 2-cylinder, water cooled gasoline engine |  |
|  | Bore & Stroke: | 76mm x 68mm |  |
|  | Piston displacement: | 617cc |  |
|  | Authorized output: | 40.8PS/ 30 kW |  |
|  | Fuel tank: | 18 liters |  |
|  | Fuel consumption: | 12 liters/h |  |
|  | Ignition: | Flywheel magneto (C.D.Ignition) |  |
|  | Starting system: | Starter motor and automatic recoil system |  |
|  | Lubrication: | Auto mixing (Separate oil tank) |  |
|  | Choke system: | Auto choke |  |
|  | Spark plug: | NGK BP7HS-10 |  |
|  | Light bulb: | 12V-35W |  |
|  | Battery: | 12V-16Ah/5h |  |
| Pump: | Type: | Single suction, single stage, high pressure turbine pump |  |
|  | Delivery outlet: | JIS fire thread (B-9912), 2-1/2"(65mm) male |  |
|  |  | BSP 65mm (G 2-1/2") male |  |
|  | Suction inlet: | JIS fire thread (B-9912), 3" (75mm) male |  |
|  |  | BSP 100mm (G 4") male |  |
|  | Priming: | Rotary-vane vacuum pump (Oilless type) |  |
| Pump performance: (Suction height: 3m) |  | 1700 liter/min. at 0.4Mpa |  |
|  |  | 1550 liter/min. at 0.6MPa |  |
|  |  | 1300 liter/min. at 0.8MPa |  |
|  |  | 950 liter/min. at 1.0MPa |  |
| Overall dimensions: |  | Length: 700mm x Width: 620mm x Height: 730mm |  |
| Mass: | Dry: | 85kg |  |
|  | Wet: | 101kg |  |
| Max. suction height: |  | Approx. 9m (29.5 feet) |  |